# Design of 10 TPH Semi-Automatic Batching Cattle and Poultry Feed Plant

Prepared By: Govind Narayan

Company: SG Global Solutions

Software Used: SolidWorks, AutoCAD

Year: 2025

# ACKNOWLEDGEMENT

I would like to express my sincere gratitude to the Head of Department and Managing Director of SG Global Solutions for their valuable guidance and support during the development of this project.

Their technical advice and industrial knowledge helped in understanding feed plant engineering, equipment selection, and plant layout planning.

This project provided valuable practical knowledge about industrial feed manufacturing systems and CAD-based plant design.

# ABSTRACT

This project presents the engineering design of a 10 Ton Per Hour (TPH) semi‑automatic batching cattle and poultry feed plant.

The study includes process flow development, material handling design, equipment selection, plant layout planning, and 3D modeling. AutoCAD was used to prepare the Process Flow Diagram (PFD) and plant layout, while SolidWorks was used for equipment modeling.

The objective of the project is to design an efficient feed production system capable of producing high‑quality animal feed with optimized plant space and improved material flow.

# 1. INTRODUCTION

Feed manufacturing is an essential part of modern livestock and poultry farming. High‑quality feed ensures proper nutrition, faster growth, improved milk production, and better poultry performance.

A feed plant converts agricultural raw materials such as maize, soybean meal, bran, and mineral additives into nutritionally balanced feed through processes like grinding, mixing, pelletizing, cooling, screening, and packing.

This project focuses on the design of a 10 TPH semi‑automatic batching feed plant suitable for medium to large scale feed production industries.

# 2. PROJECT OBJECTIVES

• Design a 10 TPH feed manufacturing plant

• Develop Process Flow Diagram (PFD)

• Create an efficient plant layout

• Perform batch and material balance calculations

• Select suitable machinery for production

• Develop 3D equipment models using SolidWorks

• Improve production efficiency and material handling

# 3. PLANT CAPACITY AND SPECIFICATIONS

Plant Capacity: 10 Ton Per Hour

Feed Types:

• Cattle Feed

• Poultry Feed

Plant Type:

Semi‑Automatic Batching System

Batch Size:

2 Tons

Batches per Hour:

5 batches/hour

Daily Production (8 hours):

80 Tons per day

# 4. RAW MATERIALS USED

Typical feed ingredients used in cattle and poultry feed include:

Maize – Energy source

Soybean Meal – Protein source

Wheat Bran – Fiber source

Rice Bran – Energy and fiber

Mineral Mixture – Essential minerals

Salt – Electrolyte balance

Vitamin Premix – Nutritional supplements

# 5. PROCESS FLOW DIAGRAM (PFD)

Raw Material Intake

→ Intake Conveyor

→ Weighing Hopper

→ Bucket Elevator

→ Jute Remover

→ Magnetic Separator

→ Grinder Hopper

→ Rotary Feeder

→ Hammer Mill

→ Mixer

→ Pellet Mill

→ Cooler

→ Crumbler

→ Screener

→ Finished Feed Bins

→ Packing System

# 6. GRINDING SYSTEM

Grinding is performed using a high‑capacity hammer mill.

Purpose:

• Reduce particle size

• Improve feed digestibility

• Enhance mixing efficiency

Typical hammer mill capacity used in a 10 TPH plant is between 10–12 TPH depending on raw material type.

# 7. MIXING SYSTEM

The mixing system ensures uniform blending of all feed ingredients.

Equipment Used:

Horizontal Ribbon Mixer or Paddle Mixer

Batch Size:

2 tons

Mixing Time:

3 – 5 minutes per batch

Proper mixing ensures consistent feed quality and uniform nutrient distribution.

# 8. PELLETIZING SYSTEM

Pelletizing converts mash feed into compact pellets.

Process Flow:

Mixer Discharge

→ Pellet Mill Elevator

→ Conditioner (Steam addition)

→ Pellet Mill

Pellet mills compress feed through dies to form cylindrical pellets suitable for cattle and poultry feeding.

# 9. COOLING SYSTEM

Pellets leaving the pellet mill are hot and soft.

A Counter Flow Cooler is used to:

• Reduce pellet temperature

• Remove moisture

• Increase pellet strength

# 10. CRUMBLING SYSTEM

For poultry feed production, pellets are crushed into smaller particles using a crumbler.

Crumbling is mainly required for chick feed and broiler starter feed.

Cattle feed pellets normally bypass this stage.

# 11. SCREENING SYSTEM

The screening machine separates:

• Finished pellets

• Oversized particles

• Fine powder

Fine particles are recycled back to the pellet mill to minimize raw material loss.

# 12. PACKING SYSTEM

Finished feed is packed using semi‑automatic bagging machines.

Standard bag sizes:

• 25 kg

• 50 kg

Packing includes weighing, filling, stitching, and palletizing of bags.

# 13. EQUIPMENT SPECIFICATIONS

Hammer Mill Capacity: 10 TPH

Mixer Capacity: 2 Ton Batch

Pellet Mill Capacity: 10 TPH

Cooler Capacity: 10 TPH

Crumbler Capacity: 6 TPH

Bucket Elevator Capacity: 10 TPH

Screw Conveyor Capacity: 10 TPH

# 14. POWER REQUIREMENT

Hammer Mill: 110 HP

Pellet Mill: 250 HP

Mixer: 30 HP

Cooler: 15 HP

Conveyors and Elevators: 30 HP

Total Estimated Power Requirement:

Approximately 435 HP

# 15. PLANT LAYOUT DESIGN

The plant layout ensures efficient material movement and minimal manual handling.

Major sections include:

• Raw material storage area

• Grinding section

• Batching and mixing section

• Pelletizing section

• Cooling and screening section

• Finished feed storage

• Packing section

# 16. 3D EQUIPMENT MODELING

3D equipment models were developed using SolidWorks to visualize plant equipment and assembly.

Modeled equipment includes:

• Feed Mixer

• Bucket Elevator

• Screw Conveyor

• Pellet Mill

• Hammer Mill

• Complete Feed Plant Assembly

# 17. SAFETY AND MAINTENANCE

Safety measures in feed plants include:

• Machine guarding

• Dust collection systems

• Electrical protection systems

• Worker safety training

Regular maintenance ensures reliable plant operation and longer equipment life.

# 18. ECONOMIC ANALYSIS

Estimated Project Cost:

Plant Machinery: ₹1.5 Crore

Civil Construction: ₹50 Lakh

Electrical Installation: ₹20 Lakh

Installation and Commissioning: ₹10 Lakh

Total Estimated Cost:

₹2.3 Crore

# 19. CONCLUSION

This project demonstrates the engineering design and planning of a 10 TPH semi‑automatic batching cattle and poultry feed plant.

The design integrates process flow development, equipment selection, plant layout planning, and CAD‑based modeling using SolidWorks and AutoCAD. The proposed design ensures efficient feed production, optimized material handling, and improved plant productivity.

# 20. REFERENCES

Feed Manufacturing Technology Manuals

Industrial Feed Plant Engineering Guidelines

SolidWorks Documentation

AutoCAD Documentation

Livestock Feed Processing Literature